Features

- 400Gb/s to 2x200Gb/s data rate

- 8x 50Gb/s PAM4 modulation

- Single 3.3V power supply

- Max 4.5W power dissipation each QSFP56 end

- Max 8W power dissipation OSFP end

- Compliant to CMIS V5.2

- Operating case temp Commercial: 0°C to +70 °C

- Hot pluggable

- RoHS compliant

Applications

- 400Gb/s systems

- Other optical links

Order Information

Table 1-Order Information

| Part No. | Bit Rate (Gbps) | Laser (nm) | Distance | Fiber Type | DDMI | Connector | Tempnote1 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 400G-OAOC-A2-Xm | 8x50 | 850 | 1~30m | MMF | YES | N/A | 0℃~+70℃ |
| X: 1M~30M |  |  |  |  |  |  |  |

Note: 1  Case Temperature

Absolute Maximum Ratings

Table2- Absolute Maximum Ratings

| Parameter | Symbol | Min. | Typical | Max. | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Supply Voltage | Vcc3 | -0.5 | - | +3.6 | V |  |
| Storage Temperature | Ts | -10 | - | +75 | °C |  |
| Operating Humidity | RH | +5 | - | +85 | % | 1 |

Note: 1  No condensation

Recommended Operating Conditions

Table 3- Recommended operating Conditions

| Parameter | Symbol | Min. | Typical | Max. |  | Unit | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Case Temperature | TC | 0 | - | +70 | °C |  |  |
| Power Supply Voltage | Vcc | 3.14 | 3.3 | 3.47 |  | V |  |
| Power dissipation (400G retiming on all lanes) | Pd400G | - | - | 8 |  | W | 1 |
| Power dissipation (200G retiming on all lanes) | Pd200G | - | - | 4.5 |  | W | 1 |

Note: 1  Per terminal

Electrical Characteristics

Table 4- Electrical Characteristics

| Parameter | Symbol | Unit | Min | Typ | Max | Notes |
| --- | --- | --- | --- | --- | --- | --- |
| Transmitter |  |  |  |  |  |  |
| Signaling rate (each lane) | SR | GBd | 26.5625 ± 100 ppm |  |  |  |
| Differential data input voltage per lane | Vin,pp,diff | mV | - | - | 900 |  |
| Differential termination mismatchal | - | % | - | - | 10 |  |
| Single-ended voltage tolerance range | - | V | -0.4 | - | 3.3 |  |
| DC common mode voltage | - | mV | -350 | - | 2850 |  |
| Receiver |  |  |  |  |  |  |
| Signaling rate (each lane) | SR | GBd | 26.5625 ± 100 ppm |  |  |  |
| Differential output voltage | - | mV | - | - | 900 |  |
| Differential termination mismatch | - | % | - | - | 10 |  |
| Transition time (min, 20% to 80%) | - | ps | 9.5 | - | - |  |
| DC common mode voltage | - | mV | -350 | - | 2850 |  |
| Error Bit Rate | BER | - | - | - | 2.4E-4 | Note1 |

Note: 1  PRBS31Q@26.5625Gbd PAM4

Recommended Interface Circuit

Figure 1, Recommended Interface Circuit

Pin arrangement

# OSFP end

Figure 2, Pin View for OSFP

Table 5-Pin Function Definitions for OSFP

# QSFP56 end

Figure 3, Pin View for QSFP56

Table 6-Pin Function Definitions for QSFP56

| Pin | Symbol | Name/Description | Notes |
| --- | --- | --- | --- |
| 1 | GND | Ground | 1 |
| 2 | Tx2n | Transmitter Inverted Data Input |  |
| 3 | Tx2p | Transmitter Non-Inverted Data Input |  |
| 4 | GND | Ground | 1 |
| 5 | Tx4n | Transmitter Inverted Data Input |  |
| 6 | Tx4p | Transmitter Non-Inverted Data Input |  |
| 7 | GND | Ground | 1 |
| 8 | ModSelL | Module Select |  |
| 9 | ResetL | Module Reset |  |
| 10 | Vcc Rx | +3.3V Power Supply Receiver |  |
| 11 | SCL | 2-wire serial interface clock |  |
| 12 | SDA | 2-wire serial interface data |  |
| 13 | GND | Ground | 1 |
| 14 | Rx3p | Receiver Non-Inverted Data Output |  |
| 15 | Rx3n | Receiver Inverted Data Output |  |
| 16 | GND | Ground | 1 |
| 17 | Rx1p | Receiver Non-Inverted Data Output |  |
| 18 | Rx1n | Receiver Inverted Data Output |  |
| 19 | GND | Ground | 1 |
| 20 | GND | Ground | 1 |
| 21 | Rx2n | Receiver Inverted Data Output |  |
| 22 | Rx2p | Receiver Non-Inverted Data Output |  |
| 23 | GND | Ground | 1 |
| 24 | Rx4n | Receiver Inverted Data Output |  |
| 25 | Rx4p | Receiver Non-Inverted Data Output |  |
| 26 | GND | Ground | 1 |
| 27 | ModPrsL | Module Present |  |
| 28 | IntL | Interrupt |  |
| 29 | Vcc Tx | +3.3V Power supply transmitter |  |
| 30 | Vcc1 | +3.3V Power supply |  |
| 31 | LPMode | Low Power Mode |  |
| 32 | GND | Ground | 1 |
| 33 | Tx3p | Transmitter Non-Inverted Data Input |  |
| 34 | Tx3n | Transmitter Inverted Data Input |  |
| 35 | GND | Ground | 1 |
| 36 | Tx1p | Transmitter Non-Inverted Data Input |  |
| 37 | Tx1n | Transmitter Inverted Data Input |  |
| 38 | GND | Ground | 1 |

Note: 1. Circuit ground is internally isolated from chassis ground.

Monitoring Specification

400G OSFP end Compatible with CMIS rev 5.2;

200G QSFP56 end Compatible with SFF-8636.

Mechanical

# OSFP end

# Unit mm

Figure 4,Mechanical Diagram for OSFP

# QSFP56 end

# Unit mm

|  | L | L1 | L2 | L3 | L4 | W | W1 | W2 | H | H1 | H2 | H3 | H4 | H5 | H6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Max | 72.2 | - | 128 | 4.35 | 61.4 | 18.45 | - | 6.2 | 8.6 | 12.4 | 5.35 | 2.5 | 1.6 | 2.0 | - |
| Type | 72.0 | - | - | 4.20 | 61.2 | 18.35 | - | - | 8.5 | 12.2 | 5.2 | 2.3 | 1.5 | 1.8 | 6.55 |
| Min | 68.8 | 16.5 | 124 | 4.05 | 61.0 | 18.25 | 2.2 | 5.8 | 8.4 | 12.0 | 5.05 | 2.1 | 1.3 | 1.6 | - |

Figure 5, Mechanical Diagram for QSFP56

Figure 6, Mechanical Diagram of Cable

Table 7- Cable Length

| Parameter | Value | Units |
| --- | --- | --- |
| Diameter | 3±0.2 | mm |
| Minimum bend radius | 30 | mm |
| Length tolerance | 1 m ≤length ≤ 4.5 m: +15 / -0 | cm |
|  | 5 m ≤length ≤ 14.5 m: +30 / -0 | cm |
|  | Length ≥ 15.0 m +2% / -0 | m |
| Cable color | Aqua |  |

Table 8- Breakout Cable Nominal Length

| Total Length X (Unit: m) | Breakout Point Measured from OSFP LQ (Unit: m) | Breakout Point Measured from QSFP56 LS(Unit: m) |
| --- | --- | --- |
| 1 | 0.3 | 0.7 |
| 2 | 0.6 | 1.4 |
| 3 | 1 | 2 |
| 5 | 2 | 3 |
| 7 | 4 | 3 |
| 10 | 7 | 3 |
| 15 | 12 | 3 |
| 20 | 17 | 3 |
| 25 | 22 | 3 |
| 30 | 27 | 3 |

# Table 9: Connectivity Schematic

| 400Gb/s Side | 200Gb/s Side |
| --- | --- |
|  | Port 1 |
| TX1 | RX1 |
| RX1 | TX1 |
| TX2 | RX2 |
| RX2 | TX2 |
| TX3 | RX3 |
| RX3 | TX3 |
| TX4 | RX4 |
| RX4 | TX4 |
|  | Port 2 |
| TX5 | RX1 |
| RX5 | TX1 |
| TX6 | RX2 |
| RX6 | TX2 |
| TX7 | RX3 |
| RX7 | TX3 |
| TX8 | RX4 |
| RX8 | TX4 |

# Warnings

Handling Precautions: This device is susceptible to damage as a result of electrostatic discharge (ESD).

A static free environment is highly recommended. Follow guidelines according to proper ESD procedures.

Laser Safety: Radiation emitted by laser devices can be dangerous to human eyes. Avoid eye exposure to direct or indirect radiation.